Semiconductor Device Numbering Systems

American JEDEC Numbering System

The JEDEC system, ( J oint Electron Device Engineering Council. This system

has the following format:

digit, letter, serial number, [suffix]

Digit:

The first digit designates the amount of P-N junctions in the device.

So a device starting with "2" would contain 2 P-N junctions and would most likely

be either a transistor or a FET. Common part numbers are listed below:

1. Diodes

2. Bipolar transistors or Field Effect Transistors

3. Double Gate MOSFETS, SCR's

4. Opto Couplers

Letter:

The letter is always "N", and the remaining figures contain the device serial

number.

Serial Number:

The serial number runs from 100 to 9999 and indicates nothing about the

transistor.

Suffix:

If a suffix is present then this indicates the gain group as below:

A = low gain

B = medium gain

C = high gain

No suffix = ungrouped (any gain).

So for example, 1N4001 would be a diode and 3N201 would be a double gate

MOSFET.

JIS System The Japanese Industrial Standard has the following format: digit, two letters, serial number, [suffix]

Digit: This indicates the amount of p-n junctions as in the JEDEC code.

Letters: The letters indicate the intended application for the device according to the following code:

SA: PNP HF transistor SB: PNP AF transistor

SC: NPN HF transistor SD: NPN AF transistor

SE: Diodes SF: Thyristors

SG: Gunn devices SH: UJT

SJ: P-channel FET/MOSFET SK: N-channel FET/MOSFET

SM: Triac SQ: LED

SR: Rectifier SS: Signal diodes

ST: Diodes SV: Varicaps

SZ: Zener diodes

Serial Number: The serial number runs from 10-9999.

Suffix: An optional suffix indicates type approval for use by various Japanese organizations.

European Pro-Electron Numbering System

Format: two letters, [letter], serial number, [suffix]

The 1st letter specifies the semiconductor material :

A Germanium

B Silicon

C Gallium Arsenide

R Compound Materials

The 2nd letter specifies the type of device :

A Diode, low power or signal

B Diode, variable capacitance

C Transistor, audio frequency low power

D Transistor, audio frequency power

E Diode, tunnel

F Transistor, high frequency low power

G Miscellaneous devices

H Diode, sensitive to magnetism

K Hall effect device

L Transistor, high frequency power

N Photocoupler

P Light detector

Q Light emitter

R Switching device, low power e.g. thyristor, diac, UJT etc

S Transistor, low power switching

T Switching device power, e.g: thyristor, triac, etc.

U Transistor, switching power

W Surface acoustic wave device

X Diode, multiplier, e.g. varactor

Y Diode, rectifying

Z Diode, voltage reference

Third Letter if used Indicates intention for industrial or professional applications. Usually W,X,Y or Z. Eg: BFY51.

Serial Number: The serial number runs from 100-9999.

Suffix: A,B or C = low, medium or high gain

No suffix = ungrouped (any gain).

Major manufacturers often produce their own code and numbering scheme for commercial reasons. The following abbreviations represent some of the larger Semiconductor manufacturers:

MJ: Motorolla power, metal case

MJE: Motorolla power, plastic case

MPS: Motorolla low power, plastic case

MRF: Motorolla HF, VHF and microwave transistor

RCA: RCA

RCS: RCS

TIP: Texas Instruments power transistor (plastic case)

TIPL: TI planar power transistor

TIS: TI small signal transistor (plastic case)

ZT or ZTX: Ferranti

Common examples include: TIP32A, MJE3055, ZTX302.